# PipeLine Separates™

# No. 1612-PL Log Roll

Log Roll shall be PipeLine® model No. 1612-PL in the color finish and quantity shown on the bill of materials or the project drawings.  Manufacturer, Columbia Cascade Company, 1300 SW Sixth Avenue, Suite 310, Portland OR 97201-3464 U.S.A.  Phone 503/223-1157.

GENERAL

All fabrication shall take place in an enclosed factory environment by personnel experienced in the manufacture of children's play apparatus.

1.  MATERIALS

Log Roll shall consist of two support posts, log roll, safety handrail and attachment hardware.

Support posts shall be 4-1/2" (o.d.) schedule 40 steel pipe.  Log roll shall be a Playground Equipment Grade free-of-heart-center (F.O.H.C.) Douglas fir timber selected by the equipment manufacturer for strength, durability, and appearance, lathe-turned to a minimum constant diameter of 11" and be 4'-6" in length.  It shall be mounted to heavy duty bearings allowing it to rotate freely around a 1.66 (o.d.) steel supporting shaft attached to the vertical support posts.  The safety handrail shall be 1-5/16" (o.d.)  schedule 40 steel pipe and an integral part of the unit.

2.  CONSTRUCTION

Welds shall be smooth and continuous with no gaps or pin holes.  Final product shall be free of weld spatters and burrs.

3.  FINISH

Posts and safety handrail shall be color finished with CASPAX-7™, a tough, opaque, UV resistant exterior grade polyester powder coating applied to a minimum thickness of 6 mils. Liquid, epoxy or lead-containing powder coatings are not acceptable.